DSP Education Kit

LAB 2:

Sampling, Aliasing, and Reconstruction

Issue 1.0

- Contents

1	Introduction	1

1.1	Lab Overview	1

2	Requirements	1

2.1.1	WM8994 Codec	2

3	Sampling and Aliasing–Generating Sinusoids of Arbitrary Frequency	3

3.1	Exercise	6

3.2	Observations	7

3.3	Computational costs	8

3.3.1	Exercise	8

4	Square Wave Generation Using the WM8994 Codec	9

4.1	Exercise	9

5	Step and Impulse Responses of the WM8994 DAC	10

5.1	Exercise	10

5.2	Observations	11

6	Magnitude Frequency Response of the WM8994 DAC	12

6.1	Exercise	13

6.2	Observations	14

7	Time Domain Response of the WM8731 Anti-Aliasing Filter	14

8	Plotting Sample Values Read from the WM8994 ADC Using MATLAB	16

9	Magnitude Frequency Response of the WM8994 Anti-Aliasing Filter	17

10	Comparison of the WM8994 DAC and the STM32F746G 12-BIT DAC	18

11	Emulation of the Low Pass Filtering Characteristics of the WM8994 DAC Using the 12-Bit DAC	23

11.1	Using IIR filter	23

11.2	Using FIR filter	25

12	Frequency Responses of the IIR and FIR Interpolation Filters	28

13	Estimating WM8994 Codec Bandwidth Using an Adaptive Filter	30

13.1	Exercise	32

13.2	Observations	33

14	OPTIONAL: Estimating WM8994 Codec Bandwidth Using Two STM32F746G Discovery Boards	34

14.1	Exercise	36

15	Special Features of the WM8994 Codec	36

16	Conclusions	37

# Introduction

## Lab Overview

The examples in this exercise concern the characteristics of the WM8994 codec (analog-to-digital converter (ADC) and digital-to-analog converter (DAC)) used on the STM32F746G Discovery board.

In this lab, we will:

- Examine the effect of sampling rate on the bandwidth of a digital signal processing system.

- Demonstrate the phenomenon of aliasing.

- Compare the reconstruction (of an analog output signal from a sequence of discrete sample

values) characteristics of the WM8994 DAC with those of the 12-bit DAC built into the

STM32F746G microcontroller.

# Requirements

To carry out this lab, you will need:

- An STM32F746G Discovery board

- A PC running Keil MDK-Arm

- MATLAB

- GoldWave

- An oscilloscope

- Two 3.5 mm audio jacks

- Optional: An audio frequency signal generator

- Optional: An additional STM32F746G Discovery board for two-board experiment.

### WM8994 Codec

The STM32F746G Discovery board features a Cirrus Logic WM8994 stereo audio codec, which is accessed via I2C for control and I2S, using the STM32F746G microcontroller’s serial audio interface (SAI) peripheral, for data. Analog input and output signals are accessible via two three-pole (TRS) 3.5 mm jack sockets (LINE IN (CN11) and HEADPHONE OUT (CN10)) as shown in Figure 1.

As configured for these exercises, the WM8994 converts an analog input signal into 16-bit signed integer sample values, and the DAC converts 16-bit signed integer sample values into an analog output signal.

Additionally, the WM8994 has a digital microphone interface, and the STM32F746G Discovery board provides two MEMS microphones as input devices.

Figure 1: STM32F746G Discovery board

# Sampling and Aliasing–Generating Sinusoids of Arbitrary Frequency

Program stm32f7_sine_intr.c, shown in the code snippet below, generates sinusoidal analog output waveforms via the WM8994 codec using calls to the function arm_sin_f32() to compute each sample value. The program uses interrupt-based I/O, and its sampling rate is set to 8 kHz. The frequency of the sinusoidal analog output waveform is controlled by the value assigned to the variable sine_frequency, and its amplitude is controlled by the value assigned to the variable amplitude.

// stm32f7_sine_intr.c.

#include "stm32f7_wm8994_init.h"

#include "stm32f7_display.h"

#define SOURCE_FILE_NAME "stm32f7_sine_intr.c"

#define SAMPLING_FREQ 8000

extern int16_t rx_sample_L;

extern int16_t rx_sample_R;

extern int16_t tx_sample_L;

extern int16_t tx_sample_R;

float32_t sine_frequency = 1000.0;

float32_t amplitude = 10000.0;

float32_t theta_increment;

float32_t theta = 0.0;

void BSP_AUDIO_SAI_Interrupt_CallBack()

{

// when we arrive at this interrupt service routine (callback)

// the most recent input sample values are (already) in global variables

// rx_sample_L and rx_sample_R

// this routine should write new output sample values in

// global variables tx_sample_L and tx_sample_R

theta_increment = 2*PI*sine_frequency/SAMPLING_FREQ;

theta += theta_increment;

if (theta > 2*PI) theta -= 2*PI;

BSP_LED_On(LED1);

//  tx_sample_L = (int16_t)(amplitude*sin(theta));

//  tx_sample_L = (int16_t)(amplitude*sinf(theta));

tx_sample_L = (int16_t)(amplitude*arm_sin_f32(theta));

tx_sample_R = tx_sample_L;

BSP_LED_Off(LED1);

plotSamplesIntr(tx_sample_L, 32);

return;

}

int main(void)

{

stm32f7_wm8994_init(AUDIO_FREQUENCY_8K,

IO_METHOD_INTR,

INPUT_DEVICE_INPUT_LINE_1,

OUTPUT_DEVICE_HEADPHONE,

WM8994_HP_OUT_ANALOG_GAIN_0DB,

WM8994_LINE_IN_GAIN_0DB,

WM8994_DMIC_GAIN_9DB,

SOURCE_FILE_NAME,

GRAPH);

while(1){}

}

When you run the program, you should see a start screen appear on the LCD as shown in Figure 2. Press the blue user pushbutton to continue, and you should see on the LCD a graphical representation of the first 32 discrete sample values written to the DAC. Figure 3 shows the display corresponding to an output signal frequency of 367 Hz (sine_frequency = 367.0).

Figure 2: Start screen for program stm32f7_sine_intr.c

Figure 3: Graphical representation of the first 32 sample values output by program stm32f7_sine_intr.c with value of variable sine_frequency equal to 367.0

Connect either channel of the HEADPHONE OUT output on the Discovery board to an oscilloscope and verify, using both time-domain and frequency-domain oscilloscope displays, that the output signal from the program as supplied is a 1000 Hz sinusoid (sine_frequency = 1000.0).

At each sampling instant, the program statements in the interrupt service routine function BSP_AUDIO_SAI_Interrupt_CallBack()are executed. The value of the variable theta is incremented by 2*PI*sine_frequency/SAMPLING_FREQ, and a sample value equal to the sine of that angle (theta) is computed using function arm_sin_f32() and written to the DAC. The DAC reconstructs a continuous-time signal from the discrete-time sample values that are written to it, and, in this case, the reconstructed analog output signal is a sinusoidal waveform as shown in Figure 4.

Figure 4: Analog output waveform generated using program stm32f7_sine_intr.c with a value of variable sine_frequency equal to 367.0

## Exercise

- Change the value of the variable sine_frequency in program stm32f7_sine_intr.c according to Table 1 and record the frequencies of the output signals you observe on an oscilloscope connected to HEADPHONE OUT.

| Value assigned to variable sine_frequency | Frequency of analogue output signal (Hz) |
| --- | --- |
| 367 |  |
| 2573 |  |
| 7000 |  |
| 3500 |  |
| 4500 |  |

Table 1: Frequency of analog output signal for different values of variable frequency in program stm32f7_sine_intr.c

## Observations

Each time the program is run, the LCD will display the first 32 sample values written to the DAC as shown, for example, in Figure 3. Your results should be consistent with the following two observations.

- Using program stm32f7_sine_intr.c, it is possible to generate continuous-time sinusoidal waveforms of arbitrary frequency, for example, 2573 Hz. The program makes it far easier to change the frequency of the waveform than was the case using pre-computed sample values in program stm32f7_sine_lut_intr.c in Lab 1.

- Using a sampling rate of 8 kHz, the WM8994 DAC is incapable of generating a sinusoidal output signal with a frequency greater than 4 kHz. More generally, the WM8994 DAC is incapable of generating a sinusoidal output signal with a frequency greater than half its sampling frequency.

The fact that sample values computed using a value of sine_frequency greater than half the sampling rate result in an analog output waveform with a frequency that is less than half the sampling rate is an example of aliasing.

You might also have observed that for analog output signal frequencies greater than around 1 kHz, it is difficult to make much sense of the graph of sample values shown on the LCD. Nonetheless, each of the sample sequences displayed corresponds to a near perfectly sinusoidal analog output signal.

## Computational costs

You can compare the computational costs of using alternative function calls sin(), arm_sin_f32(), and sinf() by commenting out those not used (and rebuilding the project) and using an oscilloscope to measure the duration of the pulses on GPIO pin PI1. GPIO pin PI1 is set using program statement:

BSP_LED_On(LED1);

, and reset using program statement

BSP_LED_Off(LED1);

in program stm32f7_sine_intr.c.

### Exercise

Record these pulse durations in Table 2 and compare them with the duration of the corresponding pulses generated by the program stm32f7_sine_lut_intr.c in Lab 1. The durations of the pulses correspond to the execution times of the different methods of determining the value of the next output sample. The pulses are repeated at the sampling rate (8 kHz), that is, at intervals of 0.125 ms.

| Function or program | GPIO pin PI1 pulse duration |
| --- | --- |
| Arm_sin_f32() |  |
| sinf() |  |
| sin() |  |
| stm32f7_sine_lut_intr.c |  |

Table 2: GPIO pin PI1 pulse durations measured using programs stm32f7_sine_intr.c and stm32f7_sine__lut_intr.c

Note: The lookup table method of generating sinusoidal waveforms of arbitrary frequency is widely used. Program stm32f7_sweep_intr.c illustrates this using a lookup table containing 8000 samples of one cycle of a sinusoid and varying the incremental steps taken through the table to generate a range of different frequency sinusoids. Run program stm32f7_sweep_intr.c and listen to the analog output signal using headphones.

# Square Wave Generation Using the WM8994 Codec

Consider the sample sequence:

{10000, 10000, 10000, 10000, -10000, -10000, -10000, -10000}

These are samples of a square wave with a period of eight samples, that is, a period of 1 ms and a frequency of 1 kHz (at a sampling rate of 8 kHz).

Edit program stm32f7_sine_lut_intr.c, replacing the sample values representing a sinusoid in array sine_table with the sequence given above. That is, replace the relevant line in the source file with:

sine_table[LOOPLENGTH] = {10000, 10000, 10000, 10000, -10000, -10000, -10000, -10000};

Run the program and sketch the resultant output waveform (viewed using an oscilloscope) on the axes below.

## Exercise

- Run program stm32f7_sine_lut_intr.c after the modification mentioned above and sketch the analog output waveform observed in the oscilloscope.

- How would you describe the time-domain representation of the analog output?

- Is it a square wave?

If you have an oscilloscope capable of representing signals in the frequency-domain, using this gives a useful insight into the nature of the analog output signal.

# Step and Impulse Responses of the WM8994 DAC

Program stm32f7_square_intr.c repeatedly outputs a data sequence comprising 32 consecutive values of 10000 followed by 32 consecutive values of -10000. Run the program and examine the analog output waveform using an oscilloscope connected to either channel of HEADPHONE OUT. What you are seeing might be interpreted as a 125 Hz square wave signal that has been passed through the digital-to-analog converter (reconstruction filter) in the WM8994 codec (although strictly speaking, and subtly differently, it is samples of a 125 Hz square wave signal that have been passed through the digital-to-analog converter). On the Discovery board’s LCD, you should see a graphical representation of the sample values written to the DAC.

The graphical representations of signals on the LCD deliberately show each sample value as a stick or bar to emphasize their discrete nature.

## Exercise

- After running the program as mentioned above, sketch the analog output waveform seen on the oscilloscope on the axes below and, from this, deduce the impulse response of the reconstruction filter.

- Sketch what you think the impulse response of the DAC MIGHT BE on the axes below and explain how you deduced this in the space below that figure (think of each transition in the square wave as a step)

You can view the actual impulse response of the WM8994 DAC using program stm32f7_dimpulse_intr.c, or you can use the d/dt function on an oscilloscope to differentiate the waveform generated using program stm32f7_square_intr.c and see a sequence of alternately positive and negative impulse responses.

## Observations

The analog output waveform generated by program stm32f7_square_intr.c contains frequency components up to a maximum frequency of 4 kHz. Higher frequency components (that would make the edges of the analog output waveform sharper) are missing. This may be illustrated using either the Fast Fourier Transform (FFT) function of an oscilloscope or the spectrum display in GoldWave. In Figure 5, it is apparent that output from the DAC is negligible at frequencies greater than 4 kHz. This may be considered as an insight into the magnitude frequency response of the DAC.

Figure 5: Magnitudes of the frequency components present in the analogue waveform generated using program stm32f7_square_intr.c

# Magnitude Frequency Response of the WM8994 DAC

You can get a further idea of the magnitude frequency response of the WM8994 DAC using program stm32f7_prbs_intr.c. This program uses function prbs() to generate a pseudorandom binary sequence, which contains a complete range of different frequency components at equal magnitudes. When this sequence is written to the DAC, the frequency content of the reconstructed analog output signal reflects the frequency response of the DAC.

Run the program and look at the analog output signal using the FFT function of an oscilloscope or using GoldWave. You may have to adjust the input volume level in GoldWave to see the frequency content of the signal to best effect.

On the LCD, you should see a graphical representation of the first 128 sample values being written to the DAC. A pseudorandom binary sequence is one example of a signal having a power spectral density that is constant.

The function prand() generates pseudorandom sample values using the Park-Miller algorithm (a random number generator).

Replace the program statement:

tx_sample_L = prbs(8000);

With program statement:

tx_sample_L = prand();

in program stm32f7_prbs_intr.c and build and run the program again. The frequency content of the analog output signal viewed using an oscilloscope FFT function or GoldWave should be similar to that seen before and, once again, is indicative of the frequency response of the WM8994 DAC.

In terms of its analog output signal, program stm32f7_prbs_dma.c is similar to program stm32f7_prbs_intr.c but repeatedly updates the graph shown on the LCD (using the sample values generated by function prbs()) and outputs two different forms of pseudorandom signal (generated using functions prbs() and prand()) on the right and left channels of HEADPHONE OUT.

## Exercise

- Sketch the magnitude frequency response you observe using either program on the axes below.

- Run program stm32f7_prbs_intr.c again, having changed the sampling frequency to 48 kHz (as shown below).

stm32f7_wm8994_init(AUDIO_FREQUENCY_48K,

IO_METHOD_INTR,

INPUT_DEVICE_INPUT_LINE_1,

OUTPUT_DEVICE_HEADPHONE,

WM8994_HP_OUT_ANALOG_GAIN_0DB,

WM8994_LINE_IN_GAIN_0DB,

WM8994_DMIC_GAIN_0DB,

GRAPH);

- Note in the space provided below the bandwidth of the noise signal generated:

BANDWIDTH of analog output signal generated using program stm32f7_prbs_intr.c with sampling RATE 48 kHz =

## Observations

We have demonstrated in several different ways that the WM8994 DAC cannot generate signal components having frequencies greater than half the sampling frequency. This is true no matter how we produce output sample sequences. The DAC is a low-pass filter. It follows that it is inadvisable to allow analog input signal components having frequencies greater than half the sampling frequency to be sampled. This can be achieved by passing analog input signals through a low-pass anti-aliasing filter prior to sampling by the ADC. An oversampling digital anti-aliasing filter with characteristics similar to those of the reconstruction filter in the DAC is built into the ADC in the WM8994 codec.

The subsequent examples in this lab manual illustrate the presence, and characteristics, of the anti-aliasing filter that precedes the ADC in the input signal path in the WM8994.

# Time Domain Response of the WM8731 Anti-Aliasing Filter

To investigate the step response of the anti-aliasing filter in the WM8994, connect a signal generator to the left channel of the LINE IN socket. Adjust the signal generator to give a square wave output with a frequency of 125 Hz and an amplitude of approximately 1.2 V. Run program stm32f7_loop_buf_dma.c, noting that the output signal from HEADPHONE OUT is not a perfect square wave. The input samples read from the ADC are plotted on the LCD as shown in Figure 6, and the analog input and output signals are shown in the oscilloscope trace of Figure 7. The variations in the values of the input samples read from the ADC are due to the low-pass characteristic of the anti-aliasing filter.

Figure 6: Sample values read from WM8994 ADC using program stm32f7_loop_buf_dma.c. Analog input signal is a square wave with frequency 125 Hz

Figure 7: Analog input and output waveforms observed using program stm32f7_loop_buf_dma.c. Analog input signal is a square wave with frequency 125 Hz

The action of program stm32f7_loop_buf_dma.c is represented graphically in Figure 8.

Continuous voltage waveform

observable using oscilloscope

Discrete sample values

observable on LCD

ADC

Discrete-time

Continuous-time

WM8994

Figure 8: Graphical representation of the action of program stm32f7_loop_buf_dma.c

# Plotting Sample Values Read from the WM8994 ADC Using MATLAB

To save the program output into a file and view them in MATLAB, follow these steps:

- Run the program stm32f7_loop_buf_dma.c and press the user button to start the program.

- Halt it by clicking on the Stop toolbar button in the MDK_Arm debugger.

- Type the variable name lbuffer as the Address in the debugger’s Memory 1 window. Right click on the Memory 1 window and set the displayed data type to Decimal and Float.

- To view the 128 most recent input sample values read from the ADC using MATLAB, save these to a file, using the command

SAVE <filename> <start address>, <end address>

where start address is that of array lbuffer and end address is equal to (start address + 0×200)

- Launch MATLAB and plot the contents of that file using the MATLAB function stm32f7_bar_real().

You should see something similar to the display shown in Figure 9, and this should be similar to the graph plotted on the LCD. While the LCD is a useful and quick way of viewing the input samples, importing saved data into MATLAB enables more detailed analysis.

Figure 9: Input sample values read from the ADC using program stm32f7_loop_buf_dma.c when the analog input signal is a 125 Hz square wave

# Magnitude Frequency Response of the WM8994 Anti-Aliasing Filter

The low pass characteristic of the WM8994 anti-aliasing filter can be demonstrated in a slightly different way using program stm32f7_loop_buf_dma.c. Configure the signal generator to give a sinusoidal output. Run program stm32f7_loop_buf_dma.c and look at the sample values displayed on the LCD. Increase the frequency of the sinusoidal input signal. You should find that for input frequencies greater than 4 kHz (half the sampling frequency), the sample values tend to zero. The anti-aliasing filter is blocking sinusoidal input signals with frequencies greater than half the sampling frequency.

# Comparison of the WM8994 DAC and the STM32F746G 12-BIT DAC

It’s insightful to compare the characteristics of the DAC in the WM8994 codec with those of one of the 12-bit DACs built into the STM32F746G microcontroller. The 12-bit DAC might be described as a zero-order hold DAC. When a sample is written to it, it outputs a voltage proportional to the value of that sample and maintains that output voltage until another sample is written to it.

Program stm32f7_sine_lut_DAC12_intr.c is similar to program stm32f7_sine_lut_intr.c except that in addition to writing the eight different pre-computed sample values in array sine_table[] to the WM8994 DAC, it also writes them (suitably scaled and shifted) to the 12-bit DAC.

The output of the 12-bit zero-order hold DAC is accessible via pin 17 on connector P1 on the STM32F746G Discovery board. Using an oscilloscope connected to that pin, you should see an analogue output waveform similar to that shown in Figure 10.

Figure 10: Analog output waveform generated using program stm32f7_sine_lut_DAC12_intr.c (blue: 12-bit DAC, yellow: WM8994 DAC)

Programs stm32f7_square_DAC12_intr.c, stm32f7_prbs_DAC12_intr.c, and stm32f7_dimpulse_DAC12_intr.c all write sequences of sample values to both the WM8994 and 12-bit DACs, and their analog output signals may be compared using an oscilloscope.

The outputs are shown below:

Figure 11: Analog output waveform generated by WM8994 DAC using program stm32f7_sine_lut_intr.c

Figure 12: Analog output waveform generated by 12-bit DAC using program stm32f7_sine_lut_DAC12_intr.c

Figure 13: Analog output waveform generated by WM8994 DAC using program stm32f7_square_intr.c

Figure 14: Analog output waveform generated by 12-bit DAC using program stm32f7_square_DAC12_intr.c

While it might be argued that the 12-bit DAC is better than the WM8994 DAC at generating a square wave, overall and certainly for audio signals, the characteristics of the WM8994 DAC are preferable.

Figure 15: Analog output waveform generated by WM8994 DAC using program stm32f7_dimpulse_intr.c

Figure 16: Analog output waveform generated by 12-bit DAC using program stm32f7_dimpulse_DAC12_intr.c

Figure 17: Analog output waveform generated by WM8994 DAC using program stm32f7_prbs_DAC12_intr.c

Figure 18: Analog output waveform generated by 12-bit DAC using program stm32f7_prbs_DAC12_intr.c

The analog output signals generated by program stm32f7_prbs_DAC12_intr.c shown in Figure 17 and Figure 18 illustrate clearly that while the magnitude frequency response of the WM8994 codec is close to that of an ideal low pass filter with a cut-off frequency of fs/2, the 12-bit DAC has a far less pronounced low pass magnitude frequency response. Indeed, since the impulse response of the 12-bit DAC is a rectangular pulse, as shown in Figure 16, then its magnitude frequency response is anticipated to be equal to the magnitude of a Dirichlet or aliased sinc function.

# Emulation of the Low Pass Filtering Characteristics of the WM8994 DAC Using the 12-Bit DAC

## Using IIR filter

The WM8994 DAC achieves its near-ideal low pass filtering characteristics using a combination of up-sampling and digital filtering. These processes are emulated by the program stm32f7_sine_lut_recon48_IIR_intr.c. This program generates an analog output signal using the 12-bit DAC based on eight pre-computed sample values representing one cycle of a sinusoid. These sample values are accessed at a rate of 8 kHz. However, sample values that are written to the 12-bit DAC are at the higher rate of 48 kHz. At that higher sampling rate, the pre-computed sample values are used only at every sixth sampling instant. The program uses an IIR digital filter to interpolate between those pre-computed sample values, that is, to provide five out of every six sample values written to the 12-bit DAC at 48 kHz.

The resultant analog output waveform is shown in Figure 19. You do not need to have an in-depth understanding of digital filtering to observe similarities between the performance of the 12-bit DAC and the WM8994 DAC in this section.

Figure 19: Analog output waveform generated by 12-bit DAC using program stm32f7_sine_lut_recon48_IIR_intr.c

Figure 20: Graphical representation of the first 128 sample values written to the 12-bit DAC by program stm32f7_sine_lut_recon48_IIR_intr.c

The program stm32f7_sine_lut_recon48_IIR_intr.c does not emulate the WM8994 nor does it use the same method of interpolation, but it is intended to give an insight into how it achieves its performance. The first 128 sample values written to the 12-bit DAC are shown on the LCD by the program stm32f7_sine_lut_recon48_IIR_intr.c.

Remember when looking at Figure 11 and Figure 12 that they were generated using only 8 sample values per cycle of the 1 kHz sinusoidal signal.

6

fs = 8 kHz

fs = 48 kHz

LPF

Up-sample

Interpolate

DAC

STM32F746

12-bit DAC

Figure 21: Graphical representation of the operation of program stm32f7_sine_lut_recon48_IIR_intr.c

## Using FIR filter

Program stm32f7_sine_lut_recon48_FIR_intr.c uses an FIR, rather than an IIR filter to interpolate between sample values. The output waveform shown in Figure 22 is almost indistinguishable from that shown in Figure 19. However, the LCD graphs shown in Figure 20 and Figure 23 reveal a significant difference between the transient (time-domain) behaviors of the two filters. This aspect of interpolation filter performance is illustrated explicitly in Figure 24 and Figure 25, generated using programs stm32f7_dimpulse_recon48_IIR_intr.c and stm32f7_dimpulse_recon48_FIR_intr.c, respectively.

Figure 22: Analog output waveform generated by 12-bit DAC using program stm32f7_sine_lut_recon48_FIR_intr.c

Figure 23: Graphical representation of the first 128 sample values written to the 12-bit DAC by program stm32f7_sine_lut_recon48_FIR_intr.c

The lower trace (channel 2) in Figure 24 and Figure 25 shows the impulse response of the interpolation filters (in combination with the zero-order hold characteristic of the 12-bit DAC) and the upper trace (channel 1) shows the state of GPIO pin PI1, which changes at the same time as a single non-zero sample value is input to the filter. The IIR interpolation filter exhibits less latency (the time delay between the non-zero value being input to the filter and the peak of its impulse response) than the FIR interpolation filter. This is a desirable property in an audio codec.

Figure 24: Analog output waveform generated by 12-bit DAC using program stm32f7_dimpulse_recon48_IIR_intr.c and signal output on GPIO pin PI1, which indicates time at which single non-zero sample value was written to IIR filter

Figure 25: Analog output waveform generated by 12-bit DAC using program stm32f7_dimpulse_recon48_FIR_intr.c and signal output on GPIO pin PI1, which indicates time at which single non-zero sample value was written to FIR filter

Figure 26 and Figure 27 show analog output waveforms generated using programs stm32f7_square_recon48_IIR_intr.c and stm32f7_square_recon48_FIR_intr.c, respectively. Comparing these with the analog output waveform generated using the WM8994 DAC with program stm32f7_square_intr.c, it seems plausible to suggest that the WM8994 DAC incorporates an IIR interpolation filter. Some other audio codecs, for example, the Cirrus Logic WM8731, would generate analog output waveforms closer to that shown in Figure 21|.

Figure 26: Analog output waveform generated by 12-bit DAC using program stm32f7_square_recon48_IIR_intr.c

Figure 27: Analog output waveform generated by 12-bit DAC using program stm32f7_square_recon48_FIR_intr.c

# Frequency Responses of the IIR and FIR Interpolation Filters

Even though the time-domain responses of the IIR and FIR interpolation filters are quite distinct, the magnitude frequency responses suggested by the output waveforms generated by programs stm32f7_prbs_recon48_IIR_intr.c. and stm32f7_prbs_recon48_IIR_intr.c. and shown in Figure 28 and Figure 29 are quite similar.

Figure 28: Analog output waveform generated by 12-bit DAC using program stm32f7_prbs_recon48_IIR_intr.c

Figure 29: Analog output waveform generated by 12-bit DAC using program stm32f7_prbs_recon48_FIR_intr.c

Compare these with the plot of Figure 30, which uses a WM8994 DAC. The roll-off of the magnitude frequency response of the WM8994 DAC at 4 kHz is significantly sharper than that of the 12-bit DAC with oversampling.

Figure 30: Analog output waveform generated by WM8994 DAC using program stm32f7_prbs_intr.c

# Estimating WM8994 Codec Bandwidth Using an Adaptive Filter

Yet another way of illustrating the limited bandwidth of the codec is to measure its magnitude frequency response using program stm32f7_sysid_CMSIS_intr.c. You need not understand exactly how this program works in order to use it. Effectively, it identifies the characteristics of the path between its discrete-time output and its discrete-time input (points A and B in Figure 31) using an adaptive FIR filter.

Connect HEADPHONE OUT to LINE IN on the audio card using a 3.5 mm jack to 3.5 mm jack cable. The signal path from A to B that will be identified by program stm32f7_sysid_CMSIS_intr.c comprises the series combination of the digital-to-analog and analog-to-digital converters.

Figure 31: Connection diagram for WM8994 codec bandwidth identification using program stm32f7_sysid_CMSIS_intr.c

Once the program is running, you can toggle between graphs on the LCD of:

- The adaptive filter coefficients, stored in array firCoeffs32, which correspond to the impulse response of the identified signal path, and

- The magnitude of the FFT of the adaptive filter coefficient, which corresponds to the magnitude frequency response of the identified signal path.

Each graph is updated periodically, allowing the gradual convergence of the adaptive filter to be observed. The learning rate, BETA of the adaptive filter in this program, has deliberately been set very low.

The changing values of the adaptive filter coefficients may also be viewed in a memory window in MDK-Arm as shown in Figure 34.

Figure 32: Adaptive filter coefficients displayed by program stm32f7_sysid_CMSIS_intr.c

Figure 33: Magnitude of FFT of adaptive filter coefficients displayed by program stm32f7_sysid_CMSIS_intr.c

Figure 34: Connection diagram for WM8994 codec bandwidth identification using program stm32f7_sysid_CMSIS_intr.c

## Exercise

Halt the program after a few seconds and save the values of the adaptive filter coefficients firCoeffs32 to a file using the command

SAVE <filename.dat> <start address>, <end address>

where start address is that of array firCoeffs32, and end address is equal to (start address + 0×400), and plot the contents of that file using the MATLAB function stm32f7_logfft(). The order in which the filter coefficients are stored in array firCoeffs32 is time-reversed.

- Sketch the magnitude frequency response that you observe, either on the LCD or in MATLAB, on the axes below, adding a scale to the y-axis:

## Observations

The frequency range over which the magnitude frequency response has been identified is equal to half the sampling rate of the codec. To observe the frequency response of the codec beyond half its sampling frequency, we will identify the characteristics of one system with a sampling rate of 8 kHz using a second system with a sampling rate of 16 kHz.

# OPTIONAL: Estimating WM8994 Codec Bandwidth Using Two STM32F746G Discovery Boards

If possible, you can also use two STM32F746G Discovery boards to estimate the WM8994 codec bandwidth by following these steps:

- Connect two STM32F746G Discovery boards together as shown in Figure 35, that is, connect LINE IN on one board to HEADPHONE OUT on the other and vice versa. The sampling rate used by program stm32f7_sysid_CMSIS_intr.c is determined by one of the parameters passed to function stm32f7_wm8994_init().

Figure 35: Connection diagram for WM8994 codec bandwidth identification using program stm32f7_sysid_CMSIS_intr.c and two STM32F746G Discovery boards

- In program stm32f7_sysid_CMSIS_intr.c, change the program statement

stm32f7_wm8994_init(AUDIO_FREQUENCY_8K,

IO_METHOD_INTR,

INPUT_DEVICE_INPUT_LINE_1,

OUTPUT_DEVICE_HEADPHONE,

WM8994_HP_OUT_ANALOG_GAIN_0DB,

WM8994_LINE_IN_GAIN_0DB,

WM8994_DMIC_GAIN_0DB,

SOURCE_FILE_NAME,

GRAPH);

to

stm32f7_wm8994_init(AUDIO_FREQUENCY_16K,

IO_METHOD_INTR,

INPUT_DEVICE_INPUT_LINE_1,

OUTPUT_DEVICE_HEADPHONE,

WM8994_HP_OUT_ANALOG_GAIN_0DB,

WM8994_LINE_IN_GAIN_0DB,

WM8994_DMIC_GAIN_0DB,

SOURCE_FILE_NAME,

GRAPH);

- In program stm32f7_loop_intr.c, change the program statement

stm32f7_wm8994_init(AUDIO_FREQUENCY_48K,

IO_METHOD_INTR,

INPUT_DEVICE_DIGITAL MICROPHONE_2,

OUTPUT_DEVICE_HEADPHONE,

WM8994_HP_OUT_ANALOG_GAIN_6DB,

WM8994_LINE_IN_GAIN_0DB,

WM8994_DMIC_GAIN_17DB,

SOURCE_FILE_NAME,

NOGRAPH);

to

stm32f7_wm8994_init(AUDIO_FREQUENCY_8K,

IO_METHOD_INTR,

INPUT_DEVICE_INPUT_LINE_1,

OUTPUT_DEVICE_HEADPHONE,

WM8994_HP_OUT_ANALOG_GAIN_0DB,

WM8994_LINE_IN_GAIN_0DB,

WM8994_DMIC_GAIN_0DB,

SOURCE_FILE_NAME,

NOGRAPH);

- Make sure that program stm32f7_loop_intr.c (sampling rate 8 kHz) is running on one system before running program stm32f7_sysid_CMSIS_intr.c (sampling rate 16 kHz) on the other.

- After running and halting program stm32f7_sysid_CMSIS_intr.c, save the values of the 256 adaptive filter coefficients firCoeffs32 to a file using the command

SAVE <filename> <start address>, <end address>

## Exercise

Plot the impulse and magnitude frequency responses using the MATLAB function stm32f7_logfft()and sketch the magnitude frequency response on the axes of figure below. The magnitude frequency response is observed using the program stm32f7_sysid_CMSIS_intr.c and two STM32F746G Discovery boards:

# Special Features of the WM8994 Codec

The WM8994 contains several configurable and programmable features that modify the signal path through the codec including parametric equalizer intended for speaker compensation, and selectable high pass and de-emphasis filters.

It’s beyond the scope of this laboratory exercise to explain these in detail, but you can un-comment some of the program statements in program stm32f7_sysid_CMSIS_intr.c to observe the effect of these on the characteristics of the signal path through the codec.

# Conclusions

You should now have an awareness of the limited bandwidth of digital signal processing systems and of the importance and characteristics of anti-aliasing and reconstruction filters.